Here is the complete Figure Compendium (1–58) organized by section, with detailed guidance on what experimental data each figure requires and how to extract it from your DOCR-MARL codebase.

---

Document Structure

Section	Figures	Theme

1	1–4, 17, 19, 27, 43–47	System Architecture & Power System Diagrams

2	26, 26a, 28–30, 34–36	Methodology, Algorithms & Formulations

3	7–10, 12, 16, 21, 37–38	Training Dynamics & Learning Curves

4	5–6, 43–47	Performance Evaluation

5	11, 31–33, 55–57	Statistical Analysis & Ranking

6	13–14, 20, 22, 24, 47, 54	Robustness & Resilience

7	15, 23, 25, 52	Scalability & Transfer Learning

8	17–18, 51, 53	Deployment & Real-Time Implementation

9	8, 34–36, 48–50	Neural Architecture & Interpretability

10	39–42, 55–56	Ablation & Sensitivity Analysis

11	49, 52, 54, 56–58	Supplementary Analysis

Appendix	—	Master Experiment Run Matrix

---

Section 1: System Architecture & Power System Diagrams

Figure 1 — High-Level System Architecture

- Data: System component topology, software stack layers, data interfaces.

- How: Create manually in draw.io/Visio/LaTeX TikZ. No experiment needed. Reference actual modules: `env/opendss_interface.py`, `agents/mappo_safe_ewc.py`, `comm/gnn_layer.py`, `deploy/iec61850_bridge.py`.

Figure 2 — Data Flow Architecture (Single Training Step)

- Data: Per-step tensors: observation shape, GNN embedding dims, action dimensions, reward scalar, constraint vector, EWC gradient norms.

- How: Insert logging hooks in `train.py` at each step. Run 1 episode with DEBUG logging. Use TensorBoard or custom logger. Post-process into sequence diagram.

Figure 3 — IEEE 8-Bus Test System with DOCR Placement (0% PV)

- Data: Bus coordinates, line impedances (R, X), DOCR locations (primary/backup pairs), CT/VT placements.

- How: Export from OpenDSS `.dss` file. Use `Show Buslist` and `Show Lines` commands. Plot with `networkx` + `matplotlib`. Annotate DOCR placement from `protection_zone_config.json`.

Figure 4 — IEEE 30-Bus Test System with DOCR Placement

- Data: Same as Fig 3 but for 30-bus meshed system: 41 lines, tie switches, distributed PV bus assignments.

- How: Load IEEE 30-bus `.dss` model. Extract line list and bus coordinates. Annotate DOCR pairs from `protection_matrix_30bus.csv`.

Figure 17 — IEC 61850 Deployment Architecture

- Data: Physical IEDs, Merging Units, Process Bus, Station Bus, GOOSE/SV message types, Logical Node mapping (PTOC, PDIS, RBRF).

- How: Architecture diagram — no experiment. Based on IEC 61850-7-4 and 61850-8-1 standards. Map `deploy/iec61850_bridge.py` classes to Logical Nodes.

Figure 19 — PV Inverter Fault Ride-Through Curve

- Data: Voltage-time ride-through zones, reactive current injection, active power reduction.

- How: No experiment — standards reference. Plot from IEC 61427-1:2016 and IEEE 1547-2018. Use parametric curves: `V(t)` thresholds, `Iq = k×(0.9−V)` for `V < 0.9` pu.

Figure 27 — Dec-POMDP Formulation

- Data: Mathematical set definitions: N agents, S states, A actions, O observations, P transitions, R rewards, C constraints.

- How: Conceptual figure — no experiment. Derive from formal problem definition in Chapter 3. Reference actual `state_dim`, `action_dim`, `obs_dim` from `env/config.yaml`.

Figure 43 — Relay Coordination Time-Distance Diagram

- Data: Operating time vs. electrical distance from fault. Primary zone curve, backup zone curve, CTI = 0.3s gap.

- How: For each fault location along a line, compute `t_p = TDS × (PSM^α − 1)/β` and `t_b`. Ensure `t_b − t_p ≥ 0.3s`. Use OpenDSS fault study output + relay equation parameters from learned settings. Code: `utils/coordination_diagram.py`.

Figure 44 — Fault Current Contribution Pie Chart (100% PV)

- Data: Source contributions: utility substation, inverter-based resources, synchronous DG, motor loads.

- How: Run 3-phase fault at each bus in OpenDSS with 100% PV. Record `I_fault` from each source element. Normalize to percentages. Code: `experiments/fault_contribution.py`.

Figure 45 — Daily PV Generation Profile & Adaptive Relay Response

- Data: 24-hour irradiance, PV output, fault current magnitude, learned TDS/PSM trajectories.

- How: Use NREL TMY3 data. For each hour, set PV kVA in OpenDSS, run fault study, run inference with trained policy. Log `TDS_i(t)`, `PSM_i(t)`. Code: `experiments/daily_adaptive_response.py`.

Figure 46 — Relay Operating Characteristic Curves

- Data: IEC inverse-time curves with learned (TDS, PSM) points and fault current overlays.

- How: For each learned `(TDS, PSM)`, compute `t(I) = TDS × 0.14 / ((I/PSM×I_pickup)^0.02 − 1)`. Overlay fault current points. Code: `utils/relay_curves.py`.

Figure 47 — Sympathetic Tripping Prevention Analysis

- Data: Cases where backup relay trips unnecessarily due to PV infeed.

- How: Identify 10–20 fault scenarios with high PV reverse flow. Compare traditional fixed settings vs. MARL settings. Log trip outcomes. Code: `experiments/sympathetic_trip_analysis.py`.

---

Section 2: Methodology, Algorithms & Formulations

Figure 26 — MAPPO-Safe-EWC Training Algorithm Flowchart

- Data: Flowchart nodes: init, episode loop, GNN forward, PPO update, safety projection, EWC reg, Fisher update.

- How: Manual diagram from Algorithm 1 pseudocode. Use TikZ/Visio. Reference `train.py` main loop structure.

Figure 26a — Alternative Flowchart View (CTDE Perspective)

- Data: Centralized critic path vs. decentralized actor path, GNN communication detail.

- How: Same as Fig 26 but emphasizing CTDE split. Draw from `agents/ctde_mapper.py` architecture.

Figure 28 — Safe RL Dual Approach

- Data: Primal-dual loop: policy gradient ascent, constraint evaluation, dual ascent on λ, action projection.

- How: Conceptual diagram from constrained optimization theory. Use actual λ update equation from `agents/safety_layer.py`.

Figure 29 — Elastic Weight Consolidation (EWC)

- Data: Task sequence, parameter consolidation, Fisher diagonal, regularization loss, gradient flow.

- How: Conceptual + partial experiment. Run curriculum training with EWC logging. Extract: task sequence metadata, Fisher diagonal values per layer, parameter drift `||θ − θ*||²` per task. Use `agents/ewc_regularizer.py` internal state.

Figure 30 — Algorithm 1 Pseudocode

- Data: Formatted pseudocode with line numbers, input/output, loop structure.

- How: Typeset from actual `train.py` using `algorithmic` environment. Ensure pseudocode matches implementation exactly.

Figure 34 — Neural Network Architecture Detail

- Data: Layer-by-layer: input obs → GNN (GraphAttention) → MLP trunk → policy/value heads + safety + EWC hooks.

- How: Extract from `agents/network.py` using `torchsummary` or manual layer enumeration. For GNN: `num_layers=2`, `hidden_dim=64`, `heads=4`, `dropout=0.1`. Plot with Netron or custom TikZ.

Figure 35 — Observation Space Feature Breakdown

- Data: Feature vector components: voltage, current, breaker, PV%, neighbor states, topology encoding.

- How: Log obs vector from `env/observation_wrapper.py`. Print `obs.shape` and feature names. Plot bar chart of feature groups.

Figure 36 — Action Space Bounds & Safety Projection Geometry

- Data: 2D plot: TDS ∈ [0.1, 1.0] vs. PSM ∈ [1.0, 10.0]. Feasible region (CTI ≥ 0.3s), infeasible region, projection arrows.

- How: Grid search over TDS×PSM space (≈400 points). For each pair, simulate all faults and compute min CTI. Overlay raw policy actions and projected actions. Code: `experiments/action_space_geometry.py`. (2 hours)

---

Section 3: Training Dynamics & Learning Curves

Figure 7 — Curriculum Learning Progression

- Data: Per-task performance: objective value, CTI violation count, EWC loss, transfer efficiency at task boundaries.

- How: Run curriculum training with 5 tasks (0%, 25%, 50%, 75%, 100% PV). Log per-episode: `mean_reward`, `ctis_violated`, `ewc_loss`, `task_id`. At each task switch, compute backward transfer. Use `curriculum_trainer.py` with `task_schedule` config. (2 days)

Figure 8 — Learned GNN Attention Weights

- Data: Attention coefficient matrix α_ij (7×7 for 8-bus), edge-weighted graph, correlation with electrical distance.

- How: During inference on 100% PV, extract attention weights from `comm/gnn_layer.py`. Average over 100 fault scenarios. Compute attention matrix `A = mean(α_ij)`. Also compute electrical distance `d_ij = sum(line impedances along shortest path)`. Plot heatmap and scatter `A_ij` vs. `1/d_ij`. Code: `experiments/extract_attention.py`. (30 min)

Figure 9 — CTI Violation Dynamics During Training

- Data: Per-episode CTI violation rate, Lagrange multiplier norm, safety activation frequency, cumulative violations.

- How: Run MAPPO-Safe on 100% PV task (single task). Log per episode: `num_cti_violations / total_pairs`, `mean(lambda)`, `num_projections / total_steps`, `cum_violations`. Also log unconstrained MAPPO for comparison. Use 5000 episodes. Code: `experiments/single_task_training.py`. (4 hours)

Figure 10 — Lagrange Multiplier Evolution per P/B Pair

- Data: Individual λ_ij trajectories for all primary/backup pairs, steady-state values, convergence times.

- How: Same run as Fig 9. Log per-episode `λ_ij` for all `(i,j)` pairs (14 pairs for 8-bus). Plot all trajectories. Identify fastest/slowest converging pairs and highest steady-state λ. Post-process with pandas.

Figure 12 — Convergence Curves with 95% Confidence Bands

- Data: Mean episode return ± 1.96×SE over 50 independent runs. Convergence time to 95% of final performance.

- How: Execute 50 independent training runs with different random seeds (0–49). Log episode return per run. Compute mean and standard error. Identify convergence episode. Code: `experiments/batch_runs.py`. (3 days GPU)

Figure 16 — EWC Effectiveness — Performance Drop After Task Transition

- Data: Performance on Task k after training Task k+1, with vs. without EWC. Backward/forward transfer metrics.

- How: Run two ablation experiments: (a) MAPPO-Safe-EWC (full), (b) MAPPO-Safe-NoEWC (`λ_EWC = 0`). Both with curriculum 0%→100% PV. After each task transition, evaluate on ALL previous tasks. Compute backward transfer %. Run 10 seeds each. Code: `experiments/ewc_ablation.py`. (2 days)

Figure 21 — Action Projection Efficacy During Training

- Data: Projection frequency, projection magnitude `||a_raw − a_safe||`, CTI violation rate before/after projection.

- How: In `safety_layer.py`, log: (a) binary flag: was projection applied? (b) L2 norm of projection delta, (c) CTI violation count using `a_raw` vs. `a_safe`. Aggregate per 100 episodes. Same run as Fig 9.

Figure 37 — Per-Agent Learning Curves (Individual Convergence)

- Data: Separate return curves for each of the 7 agents (8-bus) or 14 agents (30-bus).

- How: Modify logger in `train.py` to record per-agent reward. For CTDE with global reward, approximate individual contribution via marginal reward. Log `V_phi_i` loss per agent. Plot all curves. Identify slowest agent (bottleneck). Code: `experiments/per_agent_curves.py`.

Figure 38 — Attention Weight Evolution During Training

- Data: Temporal heatmap: attention weights at episodes 0, 100, 500, 1000, 5000.

- How: During training, save attention weights every 100 episodes. At each checkpoint, run 10 inference episodes and average `α_ij`. Create heatmap sequence. Compute entropy `H(α_i) = −sum_j α_ij log α_ij`. Plot entropy decay. Code: `experiments/attention_evolution.py`. (Requires minor modification to training loop)

---

Section 4: Performance Evaluation

Figure 5 — Objective Function vs. PV Penetration

- Data: Objective value (weighted operating times) at 0%, 25%, 50%, 75%, 100% PV for all algorithms.

- How: Run all algorithms on each PV level with 50 seeds: MAPPO-Safe-EWC, MAPPO-Safe, MAPPO, IPPO, QMIX, IQL, Traditional Fixed. After training, evaluate on 1000 test episodes per level. Code: `experiments/pv_sweep_evaluation.py`. (7 days GPU)

Figure 6 — CTI Violation Rate vs. PV Penetration

- Data: Percentage of P/B pairs violating CTI ≥ 0.3s per PV level, per algorithm. Severity distribution.

- How: Same evaluation runs as Fig 5. For each test episode, compute all P/B pair CTIs. Count violations: mild (0.2–0.3s), moderate (0.1–0.2s), severe (<0.1s). Code: same as Fig 5, post-process with `utils/cti_analysis.py`.

---

Section 5: Statistical Analysis & Ranking

Figure 11 — Nemenyi Critical Difference Diagram

- Data: Average rank per algorithm, critical difference bar, connected equivalent groups.

- How: Use results from Fig 5 (objective) and Fig 6 (CTI violations). For each metric, rank algorithms per seed. Compute critical difference `CD = q_α × sqrt(k(k+1)/6N)` where `k=7` algorithms, `N=50` seeds. Code: `utils/nemenyi_test.py`.

Figure 31 — Overall Performance Ranking with Critical Difference

- Data: Multi-metric aggregated ranking: objective, CTI, convergence, robustness. Weighted average.

- How: Normalize each metric to [0,1]. Apply weights: `w_obj=0.3`, `w_cti=0.3`, `w_conv=0.2`, `w_rob=0.2`. Compute weighted score per algorithm per seed. Rank. Compute CD on weighted ranks. Sensitivity analysis: vary weights ±20%. Code: `utils/multi_metric_ranking.py`.

Figure 32 — Effect Size Analysis

- Data: Cohen's d (objective), Cliff's Delta (CTI violations), confidence intervals, forest plot.

- How: From Fig 5 data: for each baseline vs. MAPPO-Safe-EWC pair, compute Cohen's d. For CTI violation rates (non-normal), compute Cliff's Delta. Bootstrap 1000 samples for 95% CI. Code: `utils/effect_size.py`.

Figure 33 — Multi-Dimensional Method Comparison

- Data: Radar chart: 8 dimensions (objective, CTI, convergence, scalability, robustness, N-1, latency, memory).

- How: For each algorithm, compute 8 metrics at 100% PV (8-bus). Normalize each metric to [0,1] across algorithms. Plot radar chart. Data sources: objective/CTI from Figs 5–6; convergence from Fig 12; robustness from Fig 13; N-1 from Fig 14; latency from Fig 18; memory from training logs. Code: `utils/radar_chart.py`.

Figure 55 — Comprehensive Method Comparison Table

- Data: Numerical table with exact values for all metrics and all methods.

- How: Aggregate all results from Figs 5, 6, 12, 13, 14, 18 into a single table. Format: mean ± std. Include statistical significance indicators from t-tests. Code: `utils/comparison_table.py`.

Figure 56 — Multi-Objective Pareto Front

- Data: Pareto frontier: operating time (speed) vs. CTI violation rate (selectivity).

- How: For each algorithm and seed, plot point `(mean_operating_time, cti_violation_rate)`. Identify non-dominated points. Connect Pareto front. Code: `utils/pareto_front.py`.

Figure 57 — Bootstrap Confidence Interval Convergence

- Data: CI width vs. number of runs. Justifies N=50 protocol.

- How: From Fig 12 data (50 runs). For subset sizes `n = 5` to `50`, bootstrap 1000 times, compute 95% CI width for mean return. Plot CI width vs. `n`. Show asymptotic convergence. Code: `utils/bootstrap_convergence.py`.

---

Section 6: Robustness & Resilience

Figure 13 — Performance Degradation under Communication Stress

- Data: Objective degradation vs. packet loss (0–50%), latency (0–500ms), bandwidth reduction.

- How: Inject communication impairments in `comm/gnn_layer.py`: (a) randomly drop `α_ij` messages with probability `p`, (b) delay messages by `uniform(0, L)` ms, (c) quantize message values to lower precision. For each level, run 100 test episodes with trained policy. Code: `experiments/communication_stress.py`. (2 hours)

Figure 14 — N-1 Contingency Impact and Recovery

- Data: Performance drop per line outage, recovery time, pre-trained vs. online adaptation.

- How: For each of 41 lines in 30-bus: (a) remove line (N-1), (b) run fault study with topology change, (c) evaluate with pre-trained policy, (d) evaluate with 50-episode online fine-tuning. Code: `experiments/n1_contingency.py`. (6 hours)

Figure 20 — Byzantine Agent Detection via Trust Score Evolution

- Data: Trust scores for honest vs. malicious agents. Detection latency, false positive/negative rates.

- How: Select 1–2 agents as Byzantine. Attack types: (a) random actions, (b) collusion, (c) false data injection. Implement in `agents/byzantine_injector.py`. Each honest agent maintains trust score `τ_j = α × consistency_score + (1−α) × historical_reliability`. Code: `experiments/byzantine_resilience.py`. (2 hours)

Figure 22 — Bandwidth vs. Performance Trade-off

- Data: Required bandwidth (kbps) vs. objective value. Message size vs. GNN depth. Compression impact.

- How: Measure bandwidth: log message bytes per step × steps per second × 8 bits. For GNN depth `L=1,2,3,4`, measure message size and objective. For compression: test INT8 quantization, top-k sparsification (keep top 50% of `α_ij`). Code: `experiments/bandwidth_tradeoff.py`. (1 hour)

Figure 24 — Sensor Robustness — Performance Under Measurement Degradation

- Data: Performance vs. Gaussian noise σ (0–20%), systematic bias, missing data rate.

- How: Modify `env/observation_wrapper.py` to add noise: (a) Gaussian: `obs_noisy = obs + N(0, σ²×obs_nominal)`, (b) Bias: `obs_biased = obs × (1+b)`, (c) Missing: randomly mask `p%` of observations. For each level, run 100 test episodes. Code: `experiments/sensor_degradation.py`. (2 hours)

Figure 54 — Adversarial Robustness — Perturbed Observation Response

- Data: Performance under FGSM/PGD adversarial perturbations on sensor readings.

- How: Implement adversarial attack on observations: (a) FGSM: `obs_adv = obs + ε × sign(∇_obs L)`, (b) PGD: iterative FGSM. Compute gradient of policy loss w.r.t. obs using PyTorch autograd. For `ε ∈ {0.01, 0.05, 0.1, 0.2}`. Run 100 test episodes per ε. Code: `experiments/adversarial_robustness.py`. (2 hours)

---

Section 7: Scalability & Transfer Learning

Figure 15 — Scalability Analysis Across Test Systems

- Data: Training time, memory, convergence episodes, communication overhead vs. network size (8, 14, 30, 57, 118 buses).

- How: Train MAPPO-Safe-EWC on multiple test systems. For each: log wall-clock time per 1000 episodes, peak GPU memory, episodes to convergence, bytes communicated per step. Plot on log-log scale. Fit power law: `y = a×n^b`. Code: `experiments/scalability_sweep.py`. (5 days)

Figure 23 — Generalization to Unseen Fault Types

- Data: Performance on high-impedance faults, evolving faults, simultaneous faults.

- How: Training fault set: bolted 3-phase faults at line midpoints. Test fault set: (a) high-impedance: `Z_f = 0.1–10 Ω`, (b) evolving: L-G → 3-phase after 100ms, (c) simultaneous: faults at two locations. For each type, run 100 test episodes. Compute generalization gap. Code: `experiments/unseen_faults.py`. (2 hours)

Figure 25 — Zero-Shot Transfer — 8-Bus → 30-Bus

- Data: Performance of 8-bus policy on 30-bus without fine-tuning. Fine-tuning efficiency comparison.

- How: Take policy trained on 8-bus (100% PV). Directly evaluate on 30-bus (100% PV) → zero-shot performance. Then fine-tune on 30-bus for 500 episodes. Compare with training from scratch. Compute transfer efficiency. Code: `experiments/zero_shot_transfer.py`. (6 hours)

Figure 52 — Digital Twin Training-to-Deployment Pipeline

- Data: End-to-end pipeline: physical → digital twin → training → validation → edge deployment → monitoring → retraining.

- How: Conceptual architecture diagram — no experiment. Reference actual pipeline in codebase.

---

Section 8: Deployment & Real-Time Implementation

Figure 18 — Inference Latency Distribution (Intel NUC i5, 1000 runs)

- Data: Histogram of inference latency. CDF. Percentiles. Phase breakdown.

- How: Deploy trained policy on target hardware (Intel NUC i5-8259U, 16GB RAM). Run 1000 consecutive inference calls. Time each phase: (a) obs preprocessing, (b) GNN forward, (c) policy forward, (d) safety projection, (e) action post-processing. Use `time.perf_counter_ns()`. Code: `deploy/benchmark_latency.py`. (1 hour)

Figure 51 — Communication Protocol Stack (OSI Layer Mapping)

- Data: OSI 7-layer mapping for GNN message passing onto IEC 61850 protocols.

- How: Conceptual diagram — no experiment. Map: Physical (Ethernet 100Mbps) → Data Link (IEC 61850 GOOSE/SV) → Network (IPv4) → Transport (TCP) → Session (TLS 1.3) → Presentation (ASN.1 BER) → Application (MARL agent messages, MMS).

Figure 53 — Model Compression & Quantization Impact

- Data: Performance retention vs. model size after INT8, pruning, knowledge distillation.

- How: Apply compression to trained policy: (a) INT8 quantization, (b) Pruning: remove 50% of weights, (c) Distillation: train smaller student network. For each, evaluate on 100 test episodes. Log: model size (MB), inference time (ms), objective, CTI violations. Code: `experiments/model_compression.py`. (2 hours)

---

Section 9: Neural Architecture & Interpretability

Figure 34 — Neural Network Architecture Detail

- Data: Layer-by-layer: input obs → GNN → MLP trunk → policy/value heads + safety + EWC hooks.

- How: Extract from `agents/network.py` using `torchsummary` or manual layer enumeration. Plot with Netron or custom TikZ.

Figure 35 — Observation Space Feature Breakdown

- Data: Feature vector components: voltage, current, breaker, PV%, neighbor states, topology encoding.

- How: Log obs vector from `env/observation_wrapper.py`. Print feature names and indices. Plot bar chart of feature groups.

Figure 36 — Action Space Bounds & Safety Projection Geometry

- Data: 2D TDS×PSM space with feasible region, infeasible region, projection arrows.

- How: Grid search over TDS×PSM space (≈400 points). For each pair, simulate all faults and compute min CTI. Overlay raw and projected actions. Code: `experiments/action_space_geometry.py`. (2 hours)

Figure 48 — SHAP Feature Importance for Relay Decisions

- Data: SHAP waterfall plots showing which observations most influence TDS/PSM decisions per agent.

- How: Use `shap` library. Background dataset: 100 random observations from test episodes. Compute SHAP values for policy network. For each agent, select 5 representative decisions. Plot SHAP waterfall plots. Code: `experiments/shap_analysis.py`. (10 min per agent)

Figure 49 — Decision Boundary Visualization (2D Projection)

- Data: t-SNE/UMAP projection of observations colored by action type.

- How: Collect 5000 observations from test episodes. Run t-SNE (perplexity=30) or UMAP (n_neighbors=15) on obs vectors to 2D. Color points by discrete action. Code: `experiments/decision_boundary.py`. (5 min)

Figure 50 — Critical P/B Pair Identification

- Data: Network diagram highlighting pairs with highest Lagrange multipliers, most frequent violations.

- How: From Fig 10 data (steady-state `λ_ij`) and Fig 6 data (violation frequency per pair). Rank pairs by: (a) `λ_ij` magnitude, (b) violation count, (c) electrical criticality. Plot network with edge thickness proportional to criticality score. Code: `utils/critical_pairs.py`.

---

Section 10: Ablation & Sensitivity Analysis

Figure 39 — Systematic Ablation Study

- Data: Bar chart: baseline MAPPO + GNN + Safety + EWC + Curriculum. Marginal contribution.

- How: Run 5 ablation configurations: (1) MAPPO only, (2) MAPPO + GNN, (3) MAPPO + GNN + Safety, (4) MAPPO + GNN + Safety + EWC, (5) full + Curriculum. Each with 10 seeds on 8-bus 100% PV. Code: `experiments/ablation_study.py`. (3 days)

Figure 40 — Hyperparameter Sensitivity Heatmap

- Data: Grid: `λ_EWC × α_λ × d_h`. Performance as heatmap.

- How: Grid search over: `λ_EWC ∈ [1e2, 1e3, 1e4, 1e5, 1e6]`, `α_λ ∈ [1e-5, 1e-4, 1e-3, 1e-2]`, `d_h ∈ [16, 32, 64, 128]`. For each combination, run 3 seeds (reduced 2000 episodes for speed). Code: `experiments/hyperparameter_grid.py`. (5 days)

Figure 41 — Sample Efficiency Comparison

- Data: Performance vs. total environment steps (not episodes) for all methods.

- How: Log cumulative environment steps during training. For each method, plot objective vs. steps. Compute AUC as sample efficiency metric. Code: `utils/sample_efficiency.py`. Uses same data as Fig 12.

Figure 42 — Warm-Start vs. Cold-Start Initialization

- Data: Training from random init vs. warm-start with classical optimization solution.

- How: Compute classical DOCR settings using linear programming. Use these as initial policy parameters (warm-start) vs. random Xavier init (cold-start). Train both for 5000 episodes. Compute speedup. Code: `experiments/warm_start.py`. (1 day)

---

Section 11: Supplementary Analysis

Figure 58 — Seasonal/Weather Generalization

- Data: Performance across spring, summer, autumn, winter PV profiles without retraining.

- How: Use TMY3 seasonal profiles: extract typical day for each season. For each hour, set PV generation and run inference with trained policy (trained on summer peak). Compute seasonal degradation. Code: `experiments/seasonal_generalization.py`. (1 hour)

---

Appendix: Master Experiment Run Matrix

Figure	Category	Runs Needed	Experiment	Est. Time	Code Location

1	Architecture	N/A	Manual diagram	0	Design doc

2	Data Flow	1 episode	DEBUG logging	<1 min	train.py

3	Power System	N/A	OpenDSS export	0	OpenDSS + networkx

4	Power System	N/A	OpenDSS export	0	OpenDSS + networkx

5	Performance	1750 runs	PV sweep	7 days GPU	experiments/pv_sweep_evaluation.py

6	Performance	(shared)	PV sweep	(shared)	utils/cti_analysis.py

7	Training	10 runs	Curriculum	2 days	curriculum_trainer.py

8	Analysis	100 episodes	Attention extraction	30 min	experiments/extract_attention.py

9	Training	1 run	Single task log	4 hours	experiments/single_task_training.py

10	Training	(shared)	Single task log	(shared)	utils/lagrange_analysis.py

11	Statistical	(shared)	Nemenyi test	<1 min	utils/nemenyi_test.py

12	Training	50 runs	Batch runs	3 days GPU	experiments/batch_runs.py

13	Robustness	1 policy	Comm stress	2 hours	experiments/communication_stress.py

14	Robustness	41×2 evals	N-1 sweep	6 hours	experiments/n1_contingency.py

15	Scalability	5 systems	Multi-system	5 days	experiments/scalability_sweep.py

16	Continual	20 runs	EWC ablation	2 days	experiments/ewc_ablation.py

17	Deployment	N/A	Architecture	0	Manual diagram

18	Deployment	1000 inferences	Latency bench	1 hour	deploy/benchmark_latency.py

19	Standards	N/A	Standards ref	0	Manual plot

20	Security	1 policy	Byzantine test	2 hours	experiments/byzantine_resilience.py

21	Safety	(shared)	Safety log	(shared)	agents/safety_layer.py

22	Communication	1 policy	Bandwidth test	1 hour	experiments/bandwidth_tradeoff.py

23	Generalization	1 policy	Fault type eval	2 hours	experiments/unseen_faults.py

24	Robustness	1 policy	Sensor test	2 hours	experiments/sensor_degradation.py

25	Transfer	3 runs	Transfer eval	6 hours	experiments/zero_shot_transfer.py

26	Algorithm	N/A	Flowchart	0	Manual diagram

26a	Algorithm	N/A	Flowchart	0	Manual diagram

27	Formulation	N/A	Conceptual	0	Manual diagram

28	Methodology	N/A	Conceptual	0	Manual diagram

29	Methodology	(shared)	EWC log	(shared)	agents/ewc_regularizer.py

30	Algorithm	N/A	Pseudocode	0	Typeset from train.py

31	Statistical	(shared)	Multi-metric rank	<1 min	utils/multi_metric_ranking.py

32	Statistical	(shared)	Effect size	<1 min	utils/effect_size.py

33	Comparison	(shared)	Radar chart	<1 min	utils/radar_chart.py

34	Architecture	N/A	Network detail	0	Netron / manual

35	Features	N/A	Obs breakdown	0	env/observation_wrapper.py

36	Safety	8000 sims	Grid search	2 hours	experiments/action_space_geometry.py

37	Training	(shared)	Per-agent log	(shared)	experiments/per_agent_curves.py

38	Training	1 run	Attention snapshots	4 hours	experiments/attention_evolution.py

39	Ablation	50 runs	Component ablation	3 days	experiments/ablation_study.py

40	Sensitivity	240 runs	Grid search	5 days	experiments/hyperparameter_grid.py

41	Efficiency	(shared)	Step counting	(shared)	utils/sample_efficiency.py

42	Warm-start	20 runs	Init comparison	1 day	experiments/warm_start.py

43	Protection	(shared)	Coordination diag	(shared)	utils/coordination_diagram.py

44	Protection	8 faults	Fault study	10 min	experiments/fault_contribution.py

45	Operational	24 hours	Daily profile	1 hour	experiments/daily_adaptive_response.py

46	Protection	(shared)	Curve plot	(shared)	utils/relay_curves.py

47	Protection	20 cases	Sympathetic trip	30 min	experiments/sympathetic_trip_analysis.py

48	Interpretability	1 policy	SHAP compute	10 min/agent	experiments/shap_analysis.py

49	Interpretability	5000 obs	t-SNE/UMAP	5 min	experiments/decision_boundary.py

50	Criticality	(shared)	Pair ranking	(shared)	utils/critical_pairs.py

51	Deployment	N/A	Protocol stack	0	Manual diagram

52	Pipeline	N/A	Pipeline diagram	0	Manual diagram

53	Compression	3 models	Compress eval	2 hours	experiments/model_compression.py

54	Security	1 policy	Adversarial eval	2 hours	experiments/adversarial_robustness.py

55	Table	(shared)	Aggregation	<1 min	utils/comparison_table.py

56	Pareto	(shared)	Pareto front	<1 min	utils/pareto_front.py

57	Bootstrap	(shared)	CI convergence	<1 min	utils/bootstrap_convergence.py

58	Seasonal	4 profiles	Season eval	1 hour	experiments/seasonal_generalization.py

---

Recommended Execution Order

1. Phase 1 (Week 1): Figs 3, 4, 19, 26, 26a, 27, 28, 29, 30, 34, 35, 51, 52 — manual diagrams + architecture

2. Phase 2 (Week 2–3): Figs 12, 5, 6, 39, 40 — core training runs (start first, longest)

3. Phase 3 (Week 4): Figs 7, 9, 10, 16, 37, 38 — training dynamics (use Phase 2 checkpoints)

4. Phase 4 (Week 5): Figs 13, 14, 20, 22, 24, 54 — robustness (use trained policies)

5. Phase 5 (Week 6): Figs 15, 23, 25 — scalability & transfer

6. Phase 6 (Week 7): Figs 17, 18, 53 — deployment & hardware

7. Phase 7 (Week 8): Figs 43–47, 45, 46, 48–50 — protection engineering & interpretability

8. Phase 8 (Week 9): Figs 11, 31–33, 55–58 — statistical aggregation

9. Phase 9 (Week 10): Final compilation, cross-referencing, document integration

Total compute estimate: 30–40 GPU-days for the full suite. All training runs are parallelizable via SLURM or Ray cluster.

---

Download

Complete Figure 1–58 Experimental Guide (.docx):

[DOCR_MARL_Figure_1_to_58_Experimental_Guide.docx](sandbox:///mnt/agents/output/DOCR_MARL_Figure_1_to_58_Experimental_Guide.docx)